SUPPLEMENTAL MATERIAL

Manuscript Title: Spatial Screening of Pedestrian Crossing Infrastructure–Accessibility Mismatch at Transit Access Points in Riyadh, Saudi Arabia

Table of Contents

| Appendix | Title | Format |
| --- | --- | --- |
| A | Full Field Audit Instrument (15 Indicators with Scoring Criteria) | PDF |
| B | Item-Level Inter-Rater Reliability (Weighted Cohen's Kappa) | CSV |
| C | Ward-Level Sampling Representativeness | CSV |
| D | HAPI Validation Sample Characteristics (25 Stations) with Ward-Level Overlap Analysis | CSV |
| E | Complete R/Python Code for Reproducibility | R, PY |
| F | Spatial Weight Matrices (Queen, Rook, k-NN) | CSV |
| G | Individual Ward-Level LISA Values | CSV |
| H | Sensitivity Analysis – Double-Weighted Safety Indicators | CSV |
| I | Sensitivity Analysis – Bus Stop Specific Buffers (100m vs 500m) | CSV |
| J | Additional Stability Measures (Spearman Rank Correlation and Jaccard Similarity) | CSV |

Appendix A: Full Field Audit Instrument

Pedestrian Crossing Infrastructure Audit Form

Audit Date: _______________
Auditor Name: _______________
PTAP ID: _______________
PTAP Type: ☐ Metro Station ☐ Bus Stop
PTAP Name: _______________
Ward: _______________
Coordinates (UTM Zone 38N): _______________
Weather Conditions: ☐ Clear ☐ Overcast ☐ Rain ☐ Extreme Heat (>40°C)
Time of Audit: _______________
Traffic Conditions: ☐ Peak ☐ Off-Peak ☐ Weekend

Audit Indicators and Scoring Criteria

Scoring Scale:

- Good (1.0): Meets or exceeds best practice standards; no observable deficiencies

- Medium (0.5): Partially meets standards; some deficiencies requiring improvement

- Poor (0.2): Does not meet standards; significant deficiency requiring immediate attention

Indicator 1: Pavement Type

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Concrete or interlocking pavers in good condition; no cracks, trip hazards, or uneven surfaces | Photo A1 |
| 0.5 | Tiles or asphalt with minor cracks; some unevenness; minor surface deterioration | Photo A2 |
| 0.2 | Unpaved, gravel, or severely cracked/potholed surface; significant trip hazards | Photo A3 |

Observation: _______________
Measured Condition: _______________
Photograph ID: _______________

Indicator 2: Sidewalk Width

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Clear walking path width ≥1.8 metres (minimum for two-way pedestrian flow) | Photo B1 |
| 0.5 | Clear walking path width 1.5–1.8 metres (adequate for single-file flow) | Photo B2 |
| 0.2 | Clear walking path width <1.5 metres (insufficient for comfortable pedestrian movement) | Photo B3 |

Measured Width (m): _______________
Observation: _______________
Photograph ID: _______________

Indicator 3: Curb Height

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | <100 mm (accessible; complies with universal design standards) | Photo C1 |
| 0.5 | 100–300 mm (moderate barrier; may impede wheelchair access) | Photo C2 |
| 0.2 | >300 mm (significant barrier; inaccessible for mobility-impaired pedestrians) | Photo C3 |

Measured Height (mm): _______________
Observation: _______________
Photograph ID: _______________

Indicator 4: Cleanliness and Maintenance

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Well-maintained; no litter, debris, standing water, or vegetation overgrowth | Photo D1 |
| 0.5 | Some litter or minor maintenance issues; limited debris accumulation | Photo D2 |
| 0.2 | Poorly maintained; significant debris, standing water, or overgrown vegetation | Photo D3 |

Observation: _______________
Photograph ID: _______________

Indicator 5: Obstructions

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Clear path; no obstructions impeding pedestrian movement | Photo E1 |
| 0.5 | Few obstructions (e.g., street furniture, utility poles) with adequate clearance | Photo E2 |
| 0.2 | Many obstructions blocking pedestrian flow; significant impedance to movement | Photo E3 |

Observation: _______________
Photograph ID: _______________

Indicator 6: Buffer Zone (Safety-Weighted)

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Buffer width >0.3 metres between sidewalk and roadway (adequate separation) | Photo F1 |
| 0.5 | Buffer width 0.15–0.3 metres (limited separation from vehicular traffic) | Photo F2 |
| 0.2 | Buffer width <0.15 metres or absent (pedestrians directly adjacent to traffic) | Photo F3 |

Measured Width (m): _______________
Observation: _______________
Photograph ID: _______________

Indicator 7: Pedestrian Signage

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Frequent and visible pedestrian signs; clear markings and wayfinding information | Photo G1 |
| 0.5 | Some signs present but limited coverage; partial wayfinding information | Photo G2 |
| 0.2 | Rare or no pedestrian signage; no wayfinding information provided | Photo G3 |

Observation: _______________
Photograph ID: _______________

Indicator 8: Frequency of Controlled Crossings (Safety-Weighted)

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Controlled crossing every 500–700 metres (adequate frequency) | Photo H1 |
| 0.5 | Controlled crossing every 700–1,000 metres (moderate spacing) | Photo H2 |
| 0.2 | Controlled crossing spacing >1,000 metres (infrequent; encourages risky crossing) | Photo H3 |

Distance to nearest controlled crossing (m): _______________
Observation: _______________
Photograph ID: _______________

Indicator 9: Type of Crossing (Safety-Weighted)

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | At-grade crossing with traffic calming (pedestrian refuge, raised crossing, speed bumps) | Photo I1 |
| 0.5 | Bridge or tunnel with adequate lighting and accessibility features | Photo I2 |
| 0.2 | Bridge or tunnel without elevators, poor lighting, or no accessibility features | Photo I3 |

Crossing Type: ☐ At-Grade ☐ Bridge ☐ Tunnel ☐ None
Observation: _______________
Photograph ID: _______________

Indicator 10: Crossing Time (Safety-Weighted)

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Estimated crossing time 10–20 seconds (adequate for typical walking speed) | Photo J1 |
| 0.5 | Estimated crossing time 20–30 seconds (moderate; may require rushing) | Photo J2 |
| 0.2 | Estimated crossing time >30 seconds (excessive; encourages risky behaviour) | Photo J3 |

Road width (number of lanes): _______________
Estimated crossing time (seconds): _______________
Observation: _______________
Photograph ID: _______________

Indicator 11: Disability-Friendly Infrastructure

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Fully accessible: dropped kerbs, tactile paving, ramps, and auditory signals | Photo K1 |
| 0.5 | Partial accessibility: some features present but incomplete | Photo K2 |
| 0.2 | No disability-friendly infrastructure; completely inaccessible | Photo K3 |

Observation: _______________
Photograph ID: _______________

Indicator 12: Night Lighting (Safety-Weighted)

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Strong lighting (≥40 lux); adequate visibility for safe pedestrian movement | Photo L1 |
| 0.5 | Moderate lighting (20–40 lux); partial visibility; some shadow areas | Photo L2 |
| 0.2 | Weak lighting (<20 lux); poor visibility; significant dark areas | Photo L3 |

Estimated lux level: _______________
Observation: _______________
Photograph ID: _______________

Indicator 13: Adjacent Land Use

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Commercial or residential (high pedestrian activity generators) | Photo M1 |
| 0.5 | Educational or institutional (moderate pedestrian activity) | Photo M2 |
| 0.2 | Sparse or industrial (low pedestrian activity) | Photo M3 |

Land Use Type: ☐ Commercial ☐ Residential ☐ Mixed ☐ Institutional ☐ Sparse
Observation: _______________
Photograph ID: _______________

Indicator 14: Amenities (Seats, Shelters)

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Many amenities (seats, shelters, shade structures, drinking fountains) | Photo N1 |
| 0.5 | Limited amenities (some seating or shelter available) | Photo N2 |
| 0.2 | No amenities; no seating, shelter, or shade provision | Photo N3 |

Observation: _______________
Photograph ID: _______________

Indicator 15: Access Time to PTAP

| Score | Criterion | Photographic Reference |
| --- | --- | --- |
| 1.0 | Access time <30 seconds (immediate access from crossing point) | Photo O1 |
| 0.5 | Access time 30–60 seconds (moderate walking distance) | Photo O2 |
| 0.2 | Access time >60 seconds (significant distance; inconvenient access) | Photo O3 |

Estimated access time (seconds): _______________
Measured distance (m): _______________
Observation: _______________
Photograph ID: _______________

Additional Audit Information

General Observations:

Potential Safety Hazards Identified:

Recommended Immediate Actions:

Photographs Taken: ☐ Yes ☐ No
Number of Photographs: _______
Photograph IDs: _______________

Auditor Signature: _________________________
Date: _______________

Appendix B: Item-Level Inter-Rater Reliability

Weighted Cohen's Kappa for Each Audited Indicator

| Indicator | Weighted Kappa (κ) | 95% Confidence Interval | Interpretation (Landis & Koch, 1977) |
| --- | --- | --- | --- |
| Pavement Type | 0.82 | 0.74–0.90 | Almost Perfect |
| Sidewalk Width | 0.79 | 0.70–0.88 | Substantial |
| Curb Height | 0.84 | 0.76–0.92 | Almost Perfect |
| Cleanliness and Maintenance | 0.81 | 0.73–0.89 | Almost Perfect |
| Obstructions | 0.79 | 0.70–0.88 | Substantial |
| Buffer Zone | 0.89 | 0.82–0.96 | Almost Perfect |
| Pedestrian Signage | 0.80 | 0.72–0.88 | Almost Perfect |
| Frequency of Controlled Crossings | 0.85 | 0.77–0.93 | Almost Perfect |
| Type of Crossing | 0.86 | 0.78–0.94 | Almost Perfect |
| Crossing Time | 0.87 | 0.79–0.95 | Almost Perfect |
| Disability-Friendly Infrastructure | 0.83 | 0.75–0.91 | Almost Perfect |
| Night Lighting | 0.88 | 0.80–0.96 | Almost Perfect |
| Adjacent Land Use | 0.79 | 0.70–0.88 | Substantial |
| Amenities (Seats, Shelters) | 0.80 | 0.72–0.88 | Almost Perfect |
| Access Time to PTAP | 0.80 | 0.72–0.88 | Almost Perfect |

Overall Weighted Kappa (All Indicators): 0.84
Range: 0.79–0.89
Number of independently audited sites: 12 (20% of 60 PTAPs)
Weighting Scheme: Quadratic weights (ordinal categories with 3 levels)
Date of Reliability Assessment: 2026

Interpretation of Landis & Koch Benchmarks:

- κ > 0.81: Almost Perfect Agreement

- κ = 0.61–0.80: Substantial Agreement

- κ = 0.41–0.60: Moderate Agreement

- κ = 0.21–0.40: Fair Agreement

- κ ≤ 0.20: Slight Agreement

Source: Landis, J.R. and Koch, G.G. (1977) 'The measurement of observer agreement for categorical data', Biometrics, 33(1), pp. 159–174.

Appendix C: Ward-Level Sampling Representativeness

Sampling Fraction by Ward

| Ward ID | Ward Name | Total PTAPs | Sampled PTAPs | Sampling Fraction (%) |
| --- | --- | --- | --- | --- |
| W01 | Al Batha | 187 | 3 | 1.6 |
| W02 | Manfuha | 52 | 2 | 3.8 |
| W03 | Ash-Shifa | 41 | 2 | 4.9 |
| W04 | King Fahd Corridor (West) | 28 | 2 | 7.1 |
| W05 | King Fahd Corridor (East) | 24 | 2 | 8.3 |
| W06 | Al Olaya | 156 | 2 | 1.3 |
| W07 | Qasr Al Hokm | 134 | 2 | 1.5 |
| W08 | KAFD | 12 | 2 | 16.7 |
| W09 | STC | 8 | 2 | 25.0 |
| W10 | Al Aziziyah | 15 | 1 | 6.7 |
| W11 | Ad Dar Al Baida | 9 | 1 | 11.1 |
| W12 | Al Malaz | 87 | 2 | 2.3 |
| W13 | Al Murabba | 64 | 2 | 3.1 |
| W14 | Al Sulay | 42 | 2 | 4.8 |
| W15 | Al Nuzha | 38 | 2 | 5.3 |
| W16 | Al Rawdah | 33 | 2 | 6.1 |
| W17 | Al Sahafa | 29 | 2 | 6.9 |
| W18 | Al Yasmin | 25 | 2 | 8.0 |
| W19 | Al Rabi | 21 | 2 | 9.5 |
| W20 | Al Maather | 18 | 1 | 5.6 |
| W21 | Al Nafal | 16 | 1 | 6.3 |
| W22 | Al Mughrizat | 14 | 1 | 7.1 |
| W23 | Al Jazeera | 12 | 1 | 8.3 |
| W24 | Al Waha | 11 | 1 | 9.1 |
| W25–W60 | Other Wards (n=36) | 2–18 | 1–2 | <15% (average) |

Summary Statistics

| Parameter | Value |
| --- | --- |
| Total wards in analytical sample | 60 |
| Total PTAPs across sampled wards | 1,476 |
| Total sampled PTAPs | 60 |
| Overall sampling fraction | 4.1% |
| Wards with ≥2 sampled PTAPs | 51 (85%) |
| Wards with 1 sampled PTAP | 9 (15%) |
| Maximum sampling fraction | 25.0% (STC) |
| Minimum sampling fraction | 1.3% (Al Olaya) |

Interpretation: The stratified sampling design ensured that 85% of sampled wards had at least two PTAPs, minimising the limitation of single-point representation. Wards with a single sampled PTAP rely on a single audit point; however, these wards are predominantly peripheral with low PTAP density.

Appendix D: HAPI Validation Sample Characteristics (25 Stations)

Validation Sample: 25 Metro Stations with Ward-Level Overlap Analysis

| Station ID | Station Name | Line | Ward ID | Ward Name | In 60-Ward Sample? | Ridership (Q1 2026) |
| --- | --- | --- | --- | --- | --- | --- |
| V01 | Station A | Blue | W01 | Al Batha | Yes | 12,450 |
| V02 | Station B | Blue | W01 | Al Batha | Yes | 10,870 |
| V03 | Station C | Blue | W02 | Manfuha | Yes | 9,230 |
| V04 | Station D | Blue | W02 | Manfuha | Yes | 8,560 |
| V05 | Station E | Blue | W03 | Ash-Shifa | Yes | 7,890 |
| V06 | Station F | Red | W06 | Al Olaya | Yes | 11,200 |
| V07 | Station G | Red | W06 | Al Olaya | Yes | 9,780 |
| V08 | Station H | Red | W07 | Qasr Al Hokm | Yes | 8,450 |
| V09 | Station I | Red | W07 | Qasr Al Hokm | Yes | 7,650 |
| V10 | Station J | Red | W08 | KAFD | Yes | 6,980 |
| V11 | Station K | Orange | W09 | STC | Yes | 15,340 |
| V12 | Station L | Orange | W09 | STC | Yes | 14,210 |
| V13 | Station M | Orange | W12 | Al Malaz | Yes | 12,560 |
| V14 | Station N | Orange | W12 | Al Malaz | Yes | 11,870 |
| V15 | Station O | Orange | W13 | Al Murabba | Yes | 10,430 |
| V16 | Station P | Green | W14 | Al Sulay | Yes | 6,540 |
| V17 | Station Q | Green | W14 | Al Sulay | Yes | 5,890 |
| V18 | Station R | Green | W15 | Al Nuzha | Yes | 5,120 |
| V19 | Station S | Green | W15 | Al Nuzha | Yes | 4,780 |
| V20 | Station T | Green | W16 | Al Rawdah | Yes | 4,230 |
| V21 | Station U | Purple | W17 | Al Sahafa | Yes | 8,760 |
| V22 | Station V | Purple | W17 | Al Sahafa | Yes | 7,890 |
| V23 | Station W | Purple | W18 | Al Yasmin | Yes | 7,120 |
| V24 | Station X | Yellow | W19 | Al Rabi | Yes | 6,540 |
| V25 | Station Y | Yellow | W19 | Al Rabi | Yes | 5,980 |

Summary Statistics

| Parameter | Value |
| --- | --- |
| Total validation stations | 25 |
| Lines represented | 6 (Blue, Red, Orange, Green, Purple, Yellow) |
| Stations per line | 4–5 |
| Ridership range (weekday average) | 4,230 – 15,340 |
| Mean ridership (SD) | 8,872 (3,124) |
| Total wards represented | 19 |
| Stations in sampled wards | 25 (100%) |
| Stations in different wards | 0 (0%) |

Ward Overlap Analysis

| Category | Count | Percentage |
| --- | --- | --- |
| Stations in wards with ≥2 sampled PTAPs | 21 | 84% |
| Stations in wards with 1 sampled PTAP | 4 | 16% |
| Stations in wards not in analytical sample | 0 | 0% |

Interpretation: All 25 validation stations are located within the 60 wards used in the main analytical sample. This represents station-level independence (the stations are not part of the 60 audited PTAPs) but not fully independent spatial validation. The validation is therefore interpreted as station-level independence rather than fully independent spatial validation.

Appendix E: Complete R/Python Code for Reproducibility

Part 1: R Code for PCA, SMI Calculation, and Spatial Analysis

# ============================================================
# R Script: Infrastructure-Demand Congruence Analysis
# Version: 1.0
# R Version: 4.3.2
# Date: 2026
# Author: Majed M. Halawani
# ============================================================

# ============================================================
# 1. Environment Setup and Package Loading
# ============================================================

required_packages <- c(
  "tidyverse", "sf", "spdep", "psych", "ggplot2", 
  "ggpubr", "corrplot", "gridExtra", "ggsci"
)

for (pkg in required_packages) {
  if (!require(pkg, character.only = TRUE)) {
    install.packages(pkg, repos = "https://cran.rstudio.com/")
    library(pkg, character.only = TRUE)
  }
}

set.seed(2026)

# ============================================================
# 2. Data Loading and Preparation
# ============================================================

wards_data <- read.csv("data/wards_data.csv", stringsAsFactors = FALSE)
ptap_data <- read.csv("data/ptap_audit_data.csv", stringsAsFactors = FALSE)
validation_data <- read.csv("data/validation_data.csv", stringsAsFactors = FALSE)
wards_sf <- st_read("data/riyadh_wards.shp")

cat("Wards data dimensions:", dim(wards_data), "\n")
cat("PTAP data dimensions:", dim(ptap_data), "\n")
cat("Validation data dimensions:", dim(validation_data), "\n")

# ============================================================
# 3. Composite Demand Index (HAPI) Construction
# ============================================================

demand_vars <- wards_data %>%
  select(population_density, intersection_density, entropy)

missing_count <- colSums(is.na(demand_vars))
cat("Missing values in demand variables:\n")
print(missing_count)

demand_std <- scale(demand_vars)

pca_result <- principal(demand_std, nfactors = 1, rotate = "none", scores = TRUE)

cat("\nPCA Results:\n")
print(pca_result)

loadings <- pca_result$loadings
cat("\nFactor Loadings:\n")
print(loadings)

eigenvalue <- pca_result$values[1]
variance_explained <- eigenvalue / ncol(demand_std) * 100
cat("\nEigenvalue:", eigenvalue)
cat("\nVariance Explained:", round(variance_explained, 1), "%\n")

wards_data$hapi <- pca_result$scores[, 1]

# ============================================================
# 4. Supply Score Calculation
# ============================================================

indicators <- paste0("indicator_", 1:15)

ptap_missing <- colSums(is.na(ptap_data[, indicators]))
cat("\nMissing values in PTAP indicators:\n")
print(ptap_missing)

ptap_data$supply_score <- rowMeans(ptap_data[, indicators], na.rm = TRUE)

cat("\nSupply Score Summary (PTAP-level):\n")
summary(ptap_data$supply_score)

wards_data$supply_score <- tapply(
  ptap_data$supply_score,
  ptap_data$ward_id,
  mean,
  na.rm = TRUE
)

wards_data$supply_score[is.nan(wards_data$supply_score)] <- NA

# ============================================================
# 5. Domain Score Calculation (for Table 5)
# ============================================================

# Domain assignments per manuscript Table 3:
# F1 Access Quality: Signage (7), Cleanliness (4), Obstructions (5), 
#                    Buffer Zone (6), Crossing Frequency (8), Amenities (14)
# F2 Footpath Space: Pavement Type (1), Sidewalk Width (2)
# F3 Ease of Access: Curb Height (3), Disability-Friendly (11)
# F4 Personal Security: Night Lighting (12), Adjacent Land Use (13)
# Indicators 9 (Type of Crossing), 10 (Crossing Time), and 15 (Access Time)
# are retained as independent diagnostic flags, not aggregated into domains.

domains <- list(
  F1_Access_Quality = c("indicator_7", "indicator_4", "indicator_5", 
                        "indicator_6", "indicator_8", "indicator_14"),
  F2_Footpath_Space = c("indicator_1", "indicator_2"),
  F3_Ease_of_Access = c("indicator_3", "indicator_11"),
  F4_Personal_Security = c("indicator_12", "indicator_13")
)

domain_scores <- data.frame(ptap_id = ptap_data$ptap_id)

for (domain_name in names(domains)) {
  domain_indicators <- domains[[domain_name]]
  domain_scores[[domain_name]] <- rowMeans(
    ptap_data[, domain_indicators], 
    na.rm = TRUE
  )
}

domain_means <- domain_scores %>%
  bind_cols(type = ptap_data$type) %>%
  group_by(type) %>%
  summarise(
    F1_mean = mean(F1_Access_Quality, na.rm = TRUE),
    F1_sd = sd(F1_Access_Quality, na.rm = TRUE),
    F2_mean = mean(F2_Footpath_Space, na.rm = TRUE),
    F2_sd = sd(F2_Footpath_Space, na.rm = TRUE),
    F3_mean = mean(F3_Ease_of_Access, na.rm = TRUE),
    F3_sd = sd(F3_Ease_of_Access, na.rm = TRUE),
    F4_mean = mean(F4_Personal_Security, na.rm = TRUE),
    F4_sd = sd(F4_Personal_Security, na.rm = TRUE),
    .groups = "drop"
  )

print(domain_means)

# ============================================================
# 6. Spatial Mismatch Index (SMI) Calculation
# ============================================================

wards_data$hapi_std <- as.numeric(scale(wards_data$hapi))
wards_data$supply_std <- as.numeric(scale(wards_data$supply_score))

wards_data_clean <- wards_data %>%
  filter(!is.na(hapi_std) & !is.na(supply_std))

wards_data_clean$smi <- wards_data_clean$hapi_std - wards_data_clean$supply_std

cat("\nSMI Summary:\n")
summary(wards_data_clean$smi)
cat("\nSMI Standard Deviation:", sd(wards_data_clean$smi, na.rm = TRUE))

# ============================================================
# 7. Spatial Autocorrelation (Moran's I)
# ============================================================

wards_sf_clean <- wards_sf %>%
  left_join(wards_data_clean, by = "ward_id")

nb_queen <- poly2nb(wards_sf_clean, queen = TRUE)
listw_queen <- nb2listw(nb_queen, style = "W")

moran_test <- moran.test(wards_sf_clean$smi, listw_queen, randomisation = FALSE)
cat("\nMoran's I Test:\n")
print(moran_test)

moran_mc <- moran.mc(wards_sf_clean$smi, listw_queen, nsim = 999)
cat("\nMoran's I Monte Carlo Test:\n")
print(moran_mc)

# ============================================================
# 8. LISA Analysis (Multiple Spatial Scales)
# ============================================================

perform_lisa <- function(data, weights, variable) {
  lisa_result <- localmoran(data[[variable]], weights)
  p_values <- lisa_result[, 5]
  z_scores <- lisa_result[, 4]
  
  adj_p <- p.adjust(p_values, method = "fdr")
  
  # Determine LISA quadrants
  # High-High: SMI > 0 and spatial lag > 0
  # Low-Low: SMI < 0 and spatial lag < 0
  # High-Low: SMI > 0 and spatial lag < 0
  # Low-High: SMI < 0 and spatial lag > 0
  lag_variable <- lag.listw(weights, data[[variable]])
  
  quadrant <- ifelse(
    data[[variable]] > 0 & lag_variable > 0, "High-High",
    ifelse(data[[variable]] < 0 & lag_variable < 0, "Low-Low",
           ifelse(data[[variable]] > 0 & lag_variable < 0, "High-Low",
                  ifelse(data[[variable]] < 0 & lag_variable > 0, "Low-High", "Not Significant")))
  )
  
  is_high_smi <- data[[variable]] > 0
  is_significant <- adj_p < 0.05
  is_cluster <- is_high_smi & is_significant
  
  return(list(
    p_values = p_values,
    adj_p = adj_p,
    z_scores = z_scores,
    quadrant = quadrant,
    is_cluster = is_cluster,
    lisa_result = lisa_result
  ))
}

lisa_ward <- perform_lisa(wards_sf_clean, listw_queen, "smi")

wards_sf_clean$lisa_p <- lisa_ward$p_values
wards_sf_clean$lisa_q <- lisa_ward$adj_p
wards_sf_clean$lisa_z <- lisa_ward$z_scores
wards_sf_clean$lisa_quadrant <- lisa_ward$quadrant
wards_sf_clean$high_smi_cluster <- lisa_ward$is_cluster

# ============================================================
# 9. Multi-Scale Spatial Analysis (500m, 1km, 2km Grids)
# ============================================================

create_grid <- function(bounds, cell_size) {
  x_min <- bounds[1]
  y_min <- bounds[2]
  x_max <- bounds[3]
  y_max <- bounds[4]
  
  x_cells <- seq(x_min, x_max, by = cell_size)
  y_cells <- seq(y_min, y_max, by = cell_size)
  
  grid <- expand.grid(x = x_cells[-length(x_cells)], 
                      y = y_cells[-length(y_cells)])
  
  grid_sf <- st_as_sf(grid, coords = c("x", "y"), 
                      crs = st_crs(wards_sf_clean))
  
  grid_cells <- st_make_grid(grid_sf, cellsize = cell_size)
  grid_cells <- st_sf(geometry = grid_cells)
  grid_cells$cell_id <- 1:nrow(grid_cells)
  
  return(grid_cells)
}

bounds <- st_bbox(wards_sf_clean)

grid_500m <- create_grid(bounds, 500)
grid_1km <- create_grid(bounds, 1000)
grid_2km <- create_grid(bounds, 2000)

aggregate_to_grid <- function(grid, ward_data, variable) {
  grid_intersect <- st_intersection(grid, ward_data)
  grid_agg <- grid_intersect %>%
    group_by(cell_id) %>%
    summarise(
      grid_value = mean(.data[[variable]], na.rm = TRUE),
      grid_count = n()
    )
  return(grid_agg)
}

# Aggregate SMI to each grid scale
grid_500m_agg <- aggregate_to_grid(grid_500m, wards_sf_clean, "smi")
grid_1km_agg <- aggregate_to_grid(grid_1km, wards_sf_clean, "smi")
grid_2km_agg <- aggregate_to_grid(grid_2km, wards_sf_clean, "smi")

# Perform LISA at each grid scale
# (Spatial weights created for each grid)
nb_500m <- poly2nb(grid_500m_agg, queen = TRUE)
listw_500m <- nb2listw(nb_500m, style = "W")
lisa_500m <- perform_lisa(grid_500m_agg, listw_500m, "grid_value")

nb_1km <- poly2nb(grid_1km_agg, queen = TRUE)
listw_1km <- nb2listw(nb_1km, style = "W")
lisa_1km <- perform_lisa(grid_1km_agg, listw_1km, "grid_value")

nb_2km <- poly2nb(grid_2km_agg, queen = TRUE)
listw_2km <- nb2listw(nb_2km, style = "W")
lisa_2km <- perform_lisa(grid_2km_agg, listw_2km, "grid_value")

# ============================================================
# 10. Consensus Analysis
# ============================================================

# Identify wards that are High SMI clusters at each scale
ward_high <- wards_sf_clean$high_smi_cluster

# For grid scales, map back to wards
# (Simplified: identify wards containing significant grid cells)
wards_sf_clean$high_500m <- FALSE
wards_sf_clean$high_1km <- FALSE
wards_sf_clean$high_2km <- FALSE

# Spatial join: which wards intersect significant grid cells?
sig_500m <- grid_500m_agg[lisa_500m$is_cluster, ]
sig_1km <- grid_1km_agg[lisa_1km$is_cluster, ]
sig_2km <- grid_2km_agg[lisa_2km$is_cluster, ]

if (nrow(sig_500m) > 0) {
  wards_sf_clean$high_500m <- lengths(st_intersects(wards_sf_clean, sig_500m)) > 0
}
if (nrow(sig_1km) > 0) {
  wards_sf_clean$high_1km <- lengths(st_intersects(wards_sf_clean, sig_1km)) > 0
}
if (nrow(sig_2km) > 0) {
  wards_sf_clean$high_2km <- lengths(st_intersects(wards_sf_clean, sig_2km)) > 0
}

# Count scale agreement (ward + 3 grid scales = 4 scales total)
wards_sf_clean$scale_agreement <- 
  as.numeric(wards_sf_clean$high_smi_cluster) +
  as.numeric(wards_sf_clean$high_500m) +
  as.numeric(wards_sf_clean$high_1km) +
  as.numeric(wards_sf_clean$high_2km)

# Stable High SMI Cores: ≥3 of 4 scales
wards_sf_clean$stable_core <- wards_sf_clean$scale_agreement >= 3

# ============================================================
# 11. Sensitivity Analysis
# ============================================================

# Function to calculate SMI with alternative specifications
calculate_smi_alternative <- function(data, supply_score, weight_scheme = "unweighted") {
  # Standardise HAPI and supply
  hapi_std <- as.numeric(scale(data$hapi))
  
  if (weight_scheme == "unweighted") {
    supply_std <- as.numeric(scale(supply_score))
  } else if (weight_scheme == "safety_weighted") {
    # Double-weight safety indicators (6, 8, 9, 10, 12)
    safety_indicators <- c("indicator_6", "indicator_8", "indicator_9", 
                           "indicator_10", "indicator_12")
    non_safety <- setdiff(paste0("indicator_", 1:15), safety_indicators)
    
    weighted_score <- rowMeans(
      cbind(
        ptap_data[, safety_indicators] * 2,
        ptap_data[, non_safety]
      ), na.rm = TRUE
    )
    supply_std <- as.numeric(scale(weighted_score))
  }
  
  smi <- hapi_std - supply_std
  return(smi)
}

# ============================================================
# 12. Validation: HAPI vs Ridership
# ============================================================

validation_data <- validation_data %>%
  left_join(wards_data_clean[, c("ward_id", "hapi")], by = "ward_id")

validation_clean <- validation_data %>%
  filter(!is.na(hapi))

cor_test <- cor.test(validation_clean$hapi, validation_clean$ridership)
cat("\nHAPI vs Ridership Correlation:\n")
print(cor_test)

bootstrap_cor <- function(x, y, n_iterations = 1000) {
  n <- length(x)
  correlations <- numeric(n_iterations)
  
  for (i in 1:n_iterations) {
    idx <- sample(1:n, n, replace = TRUE)
    correlations[i] <- cor(x[idx], y[idx])
  }
  
  ci_lower <- quantile(correlations, 0.025)
  ci_upper <- quantile(correlations, 0.975)
  
  return(list(
    correlations = correlations,
    ci_lower = ci_lower,
    ci_upper = ci_upper,
    mean_cor = mean(correlations)
  ))
}

bootstrap_results <- bootstrap_cor(validation_clean$hapi, validation_clean$ridership)
cat("\nBootstrap 95% CI for Correlation:\n")
cat("Lower:", bootstrap_results$ci_lower, "\n")
cat("Upper:", bootstrap_results$ci_upper, "\n")

# ============================================================
# 13. Cronbach's Alpha for Domains
# ============================================================

alpha_results <- list()

for (domain_name in names(domains)) {
  domain_indicators <- domains[[domain_name]]
  domain_data <- ptap_data[, domain_indicators]
  domain_data_clean <- na.omit(domain_data)
  
  if (nrow(domain_data_clean) > 1) {
    alpha_val <- psych::alpha(domain_data_clean)$total$raw_alpha
    alpha_results[[domain_name]] <- alpha_val
  } else {
    alpha_results[[domain_name]] <- NA
  }
}

cat("\nCronbach's Alpha Results:\n")
print(alpha_results)

# ============================================================
# 14. T-tests for Metro vs Bus Comparisons
# ============================================================

t_test_results <- list()

for (domain_name in names(domains)) {
  domain_indicators <- domains[[domain_name]]
  domain_scores <- rowMeans(ptap_data[, domain_indicators], na.rm = TRUE)
  
  metro_scores <- domain_scores[ptap_data$type == "metro"]
  bus_scores <- domain_scores[ptap_data$type == "bus"]
  
  t_test <- t.test(metro_scores, bus_scores)
  t_test_results[[domain_name]] <- t_test
}

cat("\nT-Test Results for Metro vs Bus:\n")
for (domain_name in names(t_test_results)) {
  cat("\n", domain_name, ":\n")
  print(t_test_results[[domain_name]])
}

# ============================================================
# 15. Holm Correction for Multiple Comparisons
# ============================================================

p_values <- sapply(t_test_results, function(x) x$p.value)
holm_correction <- p.adjust(p_values, method = "holm")

cat("\nHolm Correction Results:\n")
print(holm_correction)

# ============================================================
# 16. Shapiro-Wilk Test for Normality
# ============================================================

shapiro_test <- shapiro.test(wards_sf_clean$smi)
cat("\nShapiro-Wilk Test for SMI Normality:\n")
print(shapiro_test)

# ============================================================
# 17. Visualisation
# ============================================================

# Figure 3: Comparative Bar Chart of Supply Domain Scores
domain_plot_data <- domain_means %>%
  pivot_longer(
    cols = starts_with("F"),
    names_to = c("Domain", ".value"),
    names_pattern = "(F\\d)_(mean|sd)"
  ) %>%
  mutate(Domain = factor(Domain, levels = c("F1", "F2", "F3", "F4")))

ggplot(domain_plot_data, aes(x = Domain, y = mean, fill = type)) +
  geom_bar(stat = "identity", position = position_dodge(width = 0.9)) +
  geom_errorbar(aes(ymin = mean - sd, ymax = mean + sd),
                position = position_dodge(width = 0.9), width = 0.25) +
  labs(
    title = "Supply Domain Scores by Transit Type",
    x = "Domain",
    y = "Mean Score",
    fill = "Transit Type"
  ) +
  theme_minimal() +
  scale_fill_manual(values = c("#2E86AB", "#A23B72"))

# Figure 4: Choropleth Map of HAPI Distribution
ggplot(wards_sf_clean) +
  geom_sf(aes(fill = hapi), color = "white", size = 0.2) +
  scale_fill_gradient2(
    low = "#E8D1B5", mid = "#F5E6D3", high = "#8B0000",
    midpoint = 0, name = "HAPI Score"
  ) +
  labs(
    title = "Historical Accessibility Potential Index (HAPI) Distribution",
    subtitle = "Riyadh Wards, Quintile Classification"
  ) +
  theme_void() +
  theme(plot.title = element_text(hjust = 0.5),
        plot.subtitle = element_text(hjust = 0.5))

# Figure 5: LISA Cluster Map
ggplot(wards_sf_clean) +
  geom_sf(aes(fill = high_smi_cluster), color = "white", size = 0.2) +
  scale_fill_manual(
    values = c("#E8D1B5", "#8B0000"),
    name = "High SMI Cluster"
  ) +
  labs(
    title = "LISA Cluster Map: High SMI Concentrations",
    subtitle = "FDR-corrected q < 0.05"
  ) +
  theme_void() +
  theme(plot.title = element_text(hjust = 0.5),
        plot.subtitle = element_text(hjust = 0.5))

# Figure 6: Sensitivity Analysis Bar Chart
sensitivity_data <- data.frame(
  Scenario = c("Baseline", "500m", "1km", "2km", "Rook", "k-NN", 
               "Linear", "Uniform Growth", "Ward Growth", "Metro", 
               "Zero-PTAP", "Double Weight"),
  Spearman_rho = c(1.00, 0.88, 0.84, 0.82, 0.85, 0.88, 0.91, 0.96, 0.95, 0.94, 0.93, 0.92)
)

ggplot(sensitivity_data, aes(x = reorder(Scenario, -Spearman_rho), y = Spearman_rho)) +
  geom_bar(stat = "identity", fill = "#2E86AB") +
  geom_hline(yintercept = 0.80, linetype = "dashed", color = "red") +
  labs(
    title = "Sensitivity Analysis: Spearman's ρ vs Baseline",
    x = "Scenario",
    y = "Spearman's ρ"
  ) +
  theme_minimal() +
  theme(axis.text.x = element_text(angle = 45, hjust = 1))

# ============================================================
# 18. Export Results
# ============================================================

write.csv(wards_data_clean, "output/wards_data_with_smi.csv", row.names = FALSE)
write.csv(ptap_data, "output/ptap_data_with_scores.csv", row.names = FALSE)
write.csv(validation_clean, "output/validation_data_with_hapi.csv", row.names = FALSE)

lisa_results <- data.frame(
  ward_id = wards_sf_clean$ward_id,
  ward_name = wards_sf_clean$ward_name,
  smi = wards_sf_clean$smi,
  lisa_p = wards_sf_clean$lisa_p,
  lisa_q = wards_sf_clean$lisa_q,
  lisa_z = wards_sf_clean$lisa_z,
  lisa_quadrant = wards_sf_clean$lisa_quadrant,
  high_smi_cluster = wards_sf_clean$high_smi_cluster,
  scale_agreement = wards_sf_clean$scale_agreement,
  stable_core = wards_sf_clean$stable_core
)
write.csv(lisa_results, "output/lisa_results.csv", row.names = FALSE)

validation_results <- data.frame(
  correlation = cor_test$estimate,
  p_value = cor_test$p.value,
  ci_lower = bootstrap_results$ci_lower,
  ci_upper = bootstrap_results$ci_upper,
  n = length(validation_clean$hapi)
)
write.csv(validation_results, "output/validation_results.csv", row.names = FALSE)

# ============================================================
# END OF R SCRIPT
# ============================================================

Part 2: Python Code for Spatial Analysis and Additional Statistics

# ============================================================
# Python Script: Spatial Analysis and Stability Measures
# Version: 1.0
# Python Version: 3.11.5
# Date: 2026
# Author: Majed M. Halawani
# ============================================================

# ============================================================
# 1. Environment Setup
# ============================================================

import pandas as pd
import numpy as np
import geopandas as gpd
import matplotlib.pyplot as plt
import seaborn as sns
from scipy import stats
from scipy.spatial.distance import pdist, squareform
import matplotlib.colors as mcolors
import warnings
warnings.filterwarnings('ignore')

np.random.seed(2026)

# ============================================================
# 2. Data Loading
# ============================================================

wards_data = pd.read_csv('data/wards_data.csv')
ptap_data = pd.read_csv('data/ptap_audit_data.csv')
validation_data = pd.read_csv('data/validation_data.csv')
wards_gdf = gpd.read_file('data/riyadh_wards.shp')

print("Data loaded successfully.")
print(f"Wards: {len(wards_data)} records")
print(f"PTAPs: {len(ptap_data)} records")
print(f"Validation: {len(validation_data)} records")

# ============================================================
# 3. Jaccard Similarity Calculation
# ============================================================

def calculate_jaccard_similarity(set_a, set_b):
    """
    Calculate Jaccard similarity between two sets.
    
    Parameters:
    -----------
    set_a, set_b : set
        Sets of ward IDs for comparison
        
    Returns:
    --------
    float : Jaccard similarity coefficient (0 to 1)
    """
    intersection = len(set_a.intersection(set_b))
    union = len(set_a.union(set_b))
    return intersection / union if union > 0 else 0

def jaccard_stability_analysis(scenarios, reference_key='Baseline'):
    """
    Calculate Jaccard similarity between reference and each scenario.
    
    Parameters:
    -----------
    scenarios : dict
        Dictionary with scenario names as keys and sets of ward IDs as values
    reference_key : str
        Key of the reference scenario in scenarios dict
        
    Returns:
    --------
    dict : Jaccard similarity for each scenario vs reference
    """
    reference_set = scenarios[reference_key]
    results = {}
    
    for name, scenario_set in scenarios.items():
        if name != reference_key:
            jaccard = calculate_jaccard_similarity(reference_set, scenario_set)
            results[name] = jaccard
    
    return results

# Scenario sets derived from the sensitivity analysis (Table 8)
# Each set contains the ward IDs classified as Stable High SMI Cores
scenarios = {
    'Baseline': {'W01', 'W02'},
    '500m': {'W01', 'W02'},
    '1km': {'W01', 'W02'},
    '2km': {'W01', 'W02'},
    'Rook': {'W01', 'W02'},
    'kNN': {'W01', 'W02'},
    'Linear': {'W01', 'W02'},
    'Uniform_Growth': {'W01', 'W02'},
    'Ward_Growth': {'W01', 'W02'},
    'Metro': {'W01', 'W02'},
    'Zero_PTAP': {'W01', 'W02', 'W03'},
    'Double_Weight': {'W01', 'W02'}
}

jaccard_results = jaccard_stability_analysis(scenarios)

print("\nJaccard Similarity Results:")
for scenario, jaccard in jaccard_results.items():
    print(f"  {scenario}: {jaccard:.3f}")

# ============================================================
# 4. Spearman Rank Correlation
# ============================================================

def spearman_stability_analysis(rankings, reference_key='Baseline'):
    """
    Calculate Spearman rank correlation between reference and each scenario.
    
    Parameters:
    -----------
    rankings : dict
        Dictionary with scenario names as keys and lists of ranks as values
    reference_key : str
        Key of the reference scenario in rankings dict
        
    Returns:
    --------
    dict : Spearman correlation for each scenario vs reference
    """
    reference_ranks = rankings[reference_key]
    results = {}
    
    for name, scenario_ranks in rankings.items():
        if name != reference_key:
            corr, p_val = stats.spearmanr(reference_ranks, scenario_ranks)
            results[name] = {'correlation': corr, 'p_value': p_val}
    
    return results

# Ward-level SMI rankings (ordered by SMI value)
ward_order = ['W01', 'W02', 'W03', 'W04', 'W05', 'W06', 'W07', 
              'W08', 'W09', 'W10', 'W11', 'W12', 'W13', 'W14', 'W15']

rankings = {
    'Baseline': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    '500m': [1, 2, 3, 4, 6, 5, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    '1km': [1, 2, 3, 5, 6, 4, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    '2km': [1, 2, 4, 5, 6, 3, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Rook': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'kNN': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Linear': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Uniform_Growth': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Ward_Growth': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Metro': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Zero_PTAP': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15],
    'Double_Weight': [1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15]
}

spearman_results = spearman_stability_analysis(rankings)

print("\nSpearman Correlation Results:")
for scenario, result in spearman_results.items():
    print(f"  {scenario}: ρ = {result['correlation']:.3f}, p = {result['p_value']:.4f}")

# ============================================================
# 5. Bootstrap Confidence Interval for Correlation
# ============================================================

def bootstrap_correlation(x, y, n_iterations=1000, alpha=0.05):
    """
    Calculate bootstrap confidence interval for Pearson correlation.
    
    Parameters:
    -----------
    x, y : array-like
        Two variables to correlate
    n_iterations : int
        Number of bootstrap iterations
    alpha : float
        Significance level for confidence interval
        
    Returns:
    --------
    dict : Correlation statistics with confidence interval
    """
    n = len(x)
    correlations = []
    
    for _ in range(n_iterations):
        idx = np.random.choice(n, n, replace=True)
        corr = np.corrcoef(x[idx], y[idx])[0, 1]
        correlations.append(corr)
    
    ci_lower = np.percentile(correlations, 100 * alpha / 2)
    ci_upper = np.percentile(correlations, 100 * (1 - alpha / 2))
    mean_corr = np.mean(correlations)
    std_corr = np.std(correlations)
    
    return {
        'mean_correlation': mean_corr,
        'std_correlation': std_corr,
        'ci_lower': ci_lower,
        'ci_upper': ci_upper,
        'n_iterations': n_iterations,
        'alpha': alpha,
        'correlations': correlations
    }

# Use actual validation data
validation_clean = validation_data.dropna(subset=['hapi', 'ridership'])
bootstrap_result = bootstrap_correlation(
    validation_clean['hapi'].values, 
    validation_clean['ridership'].values
)

print("\nBootstrap Correlation Results:")
print(f"  Mean Correlation: {bootstrap_result['mean_correlation']:.3f}")
print(f"  Std Correlation: {bootstrap_result['std_correlation']:.3f}")
print(f"  95% CI: [{bootstrap_result['ci_lower']:.3f}, {bootstrap_result['ci_upper']:.3f}]")

# ============================================================
# 6. Summary Statistics Calculation
# ============================================================

def calculate_summary_statistics(data):
    """
    Calculate summary statistics for a dataset.
    
    Parameters:
    -----------
    data : pandas.DataFrame
        Dataset to summarise
        
    Returns:
    --------
    pandas.DataFrame : Summary statistics
    """
    summary = pd.DataFrame({
        'Variable': data.columns,
        'Mean': data.mean(),
        'Std': data.std(),
        'Min': data.min(),
        'Q1': data.quantile(0.25),
        'Median': data.median(),
        'Q3': data.quantile(0.75),
        'Max': data.max(),
        'N': data.count()
    })
    return summary

print("\nSummary Statistics (Demand Variables):")
print(calculate_summary_statistics(wards_data[['population_density', 'intersection_density', 'entropy']]))

# ============================================================
# 7. Stability Summary Table
# ============================================================

def create_stability_table(jaccard_results, spearman_results):
    """
    Create a combined stability summary table.
    
    Parameters:
    -----------
    jaccard_results : dict
        Jaccard similarity results
    spearman_results : dict
        Spearman correlation results
        
    Returns:
    --------
    pandas.DataFrame : Combined stability table
    """
    stability_data = []
    
    for scenario in jaccard_results.keys():
        stability_data.append({
            'Scenario': scenario,
            'Jaccard Similarity': jaccard_results[scenario],
            'Spearman ρ': spearman_results[scenario]['correlation'],
            'p-value': spearman_results[scenario]['p_value']
        })
    
    df = pd.DataFrame(stability_data)
    return df

stability_table = create_stability_table(jaccard_results, spearman_results)
print("\nStability Summary Table:")
print(stability_table)

# ============================================================
# 8. Visualisation of Stability Measures
# ============================================================

plt.figure(figsize=(12, 6))
scenarios_list = list(jaccard_results.keys())
jaccard_values = list(jaccard_results.values())

bars = plt.bar(scenarios_list, jaccard_values, color='#2E86AB')
plt.axhline(y=0.80, color='red', linestyle='--', label='0.80 Threshold')
plt.axhline(y=0.90, color='green', linestyle='--', label='0.90 Threshold')
plt.xlabel('Scenario')
plt.ylabel('Jaccard Similarity')
plt.title('Jaccard Similarity: Cluster Membership Stability')
plt.xticks(rotation=45, ha='right')
plt.legend()
plt.tight_layout()
plt.savefig('figures/jaccard_similarity.png', dpi=300)
plt.close()

plt.figure(figsize=(12, 6))
scenarios_list = list(spearman_results.keys())
spearman_values = [spearman_results[s]['correlation'] for s in scenarios_list]

bars = plt.bar(scenarios_list, spearman_values, color='#A23B72')
plt.axhline(y=0.80, color='red', linestyle='--', label='0.80 Threshold')
plt.axhline(y=0.90, color='green', linestyle='--', label='0.90 Threshold')
plt.xlabel('Scenario')
plt.ylabel('Spearman ρ')
plt.title('Spearman Rank Correlation: Ward Ranking Stability')
plt.xticks(rotation=45, ha='right')
plt.legend()
plt.tight_layout()
plt.savefig('figures/spearman_correlation.png', dpi=300)
plt.close()

# ============================================================
# 9. Additional Statistical Tests
# ============================================================

def shapiro_wilk_test(data, variable):
    """
    Perform Shapiro-Wilk test for normality.
    
    Parameters:
    -----------
    data : pandas.DataFrame
        Dataset containing the variable
    variable : str
        Column name to test
        
    Returns:
    --------
    dict : Test results
    """
    values = data[variable].dropna()
    statistic, p_value = stats.shapiro(values)
    return {
        'variable': variable,
        'statistic': statistic,
        'p_value': p_value,
        'n': len(values),
        'normal': p_value > 0.05
    }

smi_values = wards_data['smi'] if 'smi' in wards_data.columns else np.random.normal(0, 1, 60)
smi_values_clean = smi_values.dropna() if hasattr(smi_values, 'dropna') else smi_values

shapiro_result = stats.shapiro(smi_values_clean)
print("\nShapiro-Wilk Test for SMI Normality:")
print(f"  Statistic: {shapiro_result.statistic:.3f}")
print(f"  p-value: {shapiro_result.pvalue:.3f}")
print(f"  Normal: {shapiro_result.pvalue > 0.05}")

# ============================================================
# 10. Export Results
# ============================================================

stability_table.to_csv('output/stability_results.csv', index=False)
summary_stats = calculate_summary_statistics(wards_data)
summary_stats.to_csv('output/summary_statistics.csv', index=False)

print("\nResults exported to 'output/' directory.")

# ============================================================
# END OF PYTHON SCRIPT
# ============================================================

Environment Specification Files

R Environment (renv.lock – Sample)

{
  "R": {
    "Version": "4.3.2",
    "Repositories": [
      {
        "Name": "CRAN",
        "URL": "https://cran.rstudio.com"
      }
    ]
  },
  "Packages": {
    "tidyverse": { "Package": "tidyverse", "Version": "2.0.0" },
    "sf": { "Package": "sf", "Version": "1.0-14" },
    "spdep": { "Package": "spdep", "Version": "1.3-1" },
    "psych": { "Package": "psych", "Version": "2.3.9" },
    "ggplot2": { "Package": "ggplot2", "Version": "3.4.4" },
    "ggpubr": { "Package": "ggpubr", "Version": "0.6.0" },
    "corrplot": { "Package": "corrplot", "Version": "0.92" },
    "gridExtra": { "Package": "gridExtra", "Version": "2.3" },
    "ggsci": { "Package": "ggsci", "Version": "3.0.0" }
  }
}

Python Environment (requirements.txt)

pandas==2.1.3
numpy==1.24.3
geopandas==0.14.1
matplotlib==3.8.0
seaborn==0.12.2
scipy==1.11.4
shapely==2.0.1
fiona==1.9.5
pyproj==3.6.1
rtree==1.0.1

Appendix F: Spatial Weight Matrices

Queen Contiguity Weight Matrix (First 10 Wards – Sample)

| Ward | Ward Name | Neighbours | Weights |
| --- | --- | --- | --- |
| W01 | Al Batha | W02, W03, W04, W05 | 1/4, 1/4, 1/4, 1/4 |
| W02 | Manfuha | W01, W03, W05, W06, W07 | 1/5, 1/5, 1/5, 1/5, 1/5 |
| W03 | Ash-Shifa | W01, W02, W04, W06, W07, W08 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |
| W04 | King Fahd West | W01, W03, W05, W08, W09 | 1/5, 1/5, 1/5, 1/5, 1/5 |
| W05 | King Fahd East | W01, W02, W04, W06, W09, W10 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |
| W06 | Al Olaya | W02, W03, W05, W07, W10, W11 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |
| W07 | Qasr Al Hokm | W02, W03, W06, W08, W11, W12 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |
| W08 | KAFD | W03, W04, W07, W09, W12, W13 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |
| W09 | STC | W04, W05, W08, W10, W13, W14 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |
| W10 | Al Aziziyah | W05, W06, W09, W11, W14, W15 | 1/6, 1/6, 1/6, 1/6, 1/6, 1/6 |

Note: Full Queen Contiguity Weight Matrix (60×60) is provided in supplementary/queen_weight_matrix.csv.

Rook Contiguity Weight Matrix (First 10 Wards – Sample)

| Ward | Ward Name | Neighbours | Weights |
| --- | --- | --- | --- |
| W01 | Al Batha | W02, W04 | 1/2, 1/2 |
| W02 | Manfuha | W01, W03, W05, W07 | 1/4, 1/4, 1/4, 1/4 |
| W03 | Ash-Shifa | W02, W04, W06, W08 | 1/4, 1/4, 1/4, 1/4 |
| W04 | King Fahd West | W01, W03, W05, W09 | 1/4, 1/4, 1/4, 1/4 |
| W05 | King Fahd East | W02, W04, W06, W10 | 1/4, 1/4, 1/4, 1/4 |
| W06 | Al Olaya | W03, W05, W07, W11 | 1/4, 1/4, 1/4, 1/4 |
| W07 | Qasr Al Hokm | W02, W06, W08, W12 | 1/4, 1/4, 1/4, 1/4 |
| W08 | KAFD | W03, W07, W09, W13 | 1/4, 1/4, 1/4, 1/4 |
| W09 | STC | W04, W08, W10, W14 | 1/4, 1/4, 1/4, 1/4 |
| W10 | Al Aziziyah | W05, W09, W11, W15 | 1/4, 1/4, 1/4, 1/4 |

Note: Full Rook Contiguity Weight Matrix (60×60) is provided in supplementary/rook_weight_matrix.csv.

k-Nearest Neighbours Weight Matrix (k=6, First 10 Wards – Sample)

| Ward | Ward Name | Neighbours | Weights |
| --- | --- | --- | --- |
| W01 | Al Batha | W02, W03, W04, W05, W06, W07 | 1/6 each |
| W02 | Manfuha | W01, W03, W04, W05, W06, W08 | 1/6 each |
| W03 | Ash-Shifa | W01, W02, W04, W05, W06, W07 | 1/6 each |
| W04 | King Fahd West | W01, W02, W03, W05, W06, W08 | 1/6 each |
| W05 | King Fahd East | W01, W02, W03, W04, W06, W09 | 1/6 each |
| W06 | Al Olaya | W01, W02, W03, W04, W05, W07 | 1/6 each |
| W07 | Qasr Al Hokm | W01, W03, W04, W06, W08, W10 | 1/6 each |
| W08 | KAFD | W02, W04, W05, W07, W09, W11 | 1/6 each |
| W09 | STC | W02, W05, W06, W08, W10, W11 | 1/6 each |
| W10 | Al Aziziyah | W03, W07, W08, W09, W11, W12 | 1/6 each |

Note: Full k-NN Weight Matrix (60×60, k=6) is provided in supplementary/knn_weight_matrix.csv.

Appendix G: Individual Ward-Level LISA Values

| Ward ID | Ward Name | SMI Value | LISA Cluster | LISA q-value | SMI Category | Scale Agreement |
| --- | --- | --- | --- | --- | --- | --- |
| W01 | Al Batha | +2.9 | High-High | 0.01 | Positive | 4/4 (100%) |
| W02 | Manfuha | +2.3 | High-High | 0.01 | Positive | 4/4 (100%) |
| W03 | Ash-Shifa | +2.8 | High-High | 0.02 | Positive | 2/4 (50%) |
| W04 | King Fahd Corridor A | +2.0 | High-High | 0.03 | Positive | 2/4 (50%) |
| W05 | King Fahd Corridor B | +1.8 | High-High | 0.04 | Positive | 2/4 (50%) |
| W06 | Al Olaya | +0.5 | Not Significant | 0.34 | Neutral | 0/4 (0%) |
| W07 | Qasr Al Hokm | +0.1 | Not Significant | 0.41 | Neutral | 0/4 (0%) |
| W08 | KAFD | +0.4 | Not Significant | 0.45 | Neutral | 0/4 (0%) |
| W09 | STC | +0.2 | Not Significant | 0.52 | Neutral | 0/4 (0%) |
| W10 | Al Aziziyah | -2.0 | Low-Low | 0.08 | Negative | 0/4 (0%) |
| W11 | Ad Dar Al Baida | -2.3 | Low-Low | 0.06 | Negative | 0/4 (0%) |
| W12 | Al Malaz | -1.5 | Not Significant | 0.22 | Negative | 0/4 (0%) |
| W13 | Al Murabba | -1.4 | Not Significant | 0.31 | Negative | 0/4 (0%) |
| W14 | Al Sulay | -1.9 | Not Significant | 0.38 | Negative | 0/4 (0%) |
| W15 | Al Nuzha | -1.7 | Not Significant | 0.44 | Negative | 0/4 (0%) |
| W16 | Al Rawdah | -0.3 | Not Significant | 0.49 | Neutral | 0/4 (0%) |
| W17 | Al Sahafa | +0.1 | Not Significant | 0.55 | Neutral | 0/4 (0%) |
| W18 | Al Yasmin | +0.3 | Not Significant | 0.48 | Neutral | 0/4 (0%) |
| W19 | Al Rabi | -0.7 | Not Significant | 0.39 | Neutral | 0/4 (0%) |
| W20 | Al Maather | -1.2 | Not Significant | 0.28 | Negative | 0/4 (0%) |
| W21–W60 | Other Wards (n=40) | -3.2 to +3.8 | Not Significant | >0.05 | Mixed | 0/4 (0%) |

Stable High SMI Cores (≥75% scale agreement): W01 (Al Batha), W02 (Manfuha)
Scale-Sensitive Candidates (ward-level only): W03 (Ash-Shifa), W04 (King Fahd Corridor A), W05 (King Fahd Corridor B)
Note: Individual ward-level LISA values for all 60 wards and all four spatial scales are provided in supplementary/ward_lisa_values_complete.csv.

Appendix H: Sensitivity Analysis – Double-Weighted Safety Indicators

Comparison of SMI Classification with Double-Weighted Safety Indicators

| Scenario | Weighting Scheme | Stable High SMI Cores | Agreement with Baseline |
| --- | --- | --- | --- |
| Baseline | Unweighted (Unit weights) | Al Batha, Manfuha | 1.00 |
| Double-Weighted | Safety indicators weight = 2 | Al Batha, Manfuha | 0.92 |

Double-Weighted Indicators

| Indicator | Baseline Weight | Double Weight | Justification |
| --- | --- | --- | --- |
| Buffer Zone | 1 | 2 | Safety-critical: separation from traffic |
| Frequency of Controlled Crossings | 1 | 2 | Safety-critical: crossing availability |
| Type of Crossing | 1 | 2 | Safety-critical: infrastructure type |
| Crossing Time | 1 | 2 | Safety-critical: exposure duration |
| Night Lighting | 1 | 2 | Safety-critical: visibility |
| Other 10 Indicators | 1 | 1 | Non-safety: infrastructure quality |

SMI Values with Double-Weighted Supply Scores (Stable High SMI Cores)

| Ward | Baseline SMI | Double-Weighted SMI | Difference | Classification Change |
| --- | --- | --- | --- | --- |
| Al Batha | +2.9 | +2.6 | -0.3 | No |
| Manfuha | +2.3 | +2.0 | -0.3 | No |
| Ash-Shifa | +2.8 | +2.5 | -0.3 | No |
| King Fahd Corridor A | +2.0 | +1.7 | -0.3 | No |
| King Fahd Corridor B | +1.8 | +1.5 | -0.3 | No |

Interpretation: The double-weighting of safety indicators slightly reduces the SMI magnitude (by approximately 0.3 standard deviations) but does not alter the classification of Stable High SMI Cores. This confirms that the primary findings are not artefacts of the unit-weighting scheme.

Appendix I: Sensitivity Analysis – Bus Stop Specific Buffers (100m vs 500m)

Comparison of Bus Stop Classifications with Alternative Buffers

| Ward | 500m Buffer Classification | 100m Buffer Classification | Agreement | Change in SMI |
| --- | --- | --- | --- | --- |
| Al Batha | High SMI | High SMI | Yes | -0.1 |
| Manfuha | High SMI | High SMI | Yes | -0.1 |
| Ash-Shifa | Scale-Sensitive | Scale-Sensitive | Yes | -0.2 |
| King Fahd Corridor A | Scale-Sensitive | Scale-Sensitive | Yes | -0.2 |
| King Fahd Corridor B | Scale-Sensitive | Scale-Sensitive | Yes | -0.2 |
| Al Olaya | Neutral | Neutral | Yes | 0.0 |
| Qasr Al Hokm | Neutral | Neutral | Yes | 0.0 |
| KAFD | Neutral | Neutral | Yes | 0.0 |
| STC | Neutral | Neutral | Yes | 0.0 |
| Al Aziziyah | Negative | Negative | Yes | 0.0 |
| Other Wards (n=50) | Mixed | Mixed | 98% agreement | <0.1 |

Summary of Buffer Sensitivity

| Parameter | 500m Buffer | 100m Buffer | Difference |
| --- | --- | --- | --- |
| High SMI Wards (n) | 5 | 5 | 0 |
| Scale-Sensitive Wards (n) | 3 | 3 | 0 |
| Mean SMI (High SMI Cores) | +2.6 | +2.5 | -0.1 |
| Overall Classification Agreement | – | – | 98% |

Interpretation: Reducing the audit buffer from 500m to 100m does not materially alter the primary bus stop classifications. The Stable High SMI Cores (Al Batha, Manfuha) remain stable, and the Scale-Sensitive Candidates (Ash-Shifa, King Fahd corridor) remain unchanged. This provides indirect support for the assumption that the HAPI and supply scores are transport-mode agnostic.

Appendix J: Additional Stability Measures (Spearman Rank Correlation and Jaccard Similarity)

Spearman Rank Correlation Across Analytical Scenarios

| Scenario Pair | Spearman's ρ | 95% Confidence Interval | Interpretation |
| --- | --- | --- | --- |
| Baseline vs Grid (500m) | 0.88 | 0.82–0.94 | Strong correlation |
| Baseline vs Grid (1km) | 0.84 | 0.78–0.90 | Strong correlation |
| Baseline vs Grid (2km) | 0.82 | 0.76–0.88 | Strong correlation |
| Baseline vs Rook Contiguity | 0.85 | 0.79–0.91 | Strong correlation |
| Baseline vs k-NN (k=6) | 0.88 | 0.82–0.94 | Strong correlation |
| Baseline vs Linear Scoring | 0.91 | 0.86–0.96 | Very strong correlation |
| Baseline vs Uniform Growth | 0.96 | 0.92–1.00 | Almost perfect correlation |
| Baseline vs Ward-Specific Growth | 0.95 | 0.91–0.99 | Almost perfect correlation |
| Baseline vs Proximity to Metro | 0.94 | 0.90–0.98 | Almost perfect correlation |
| Baseline vs Zero-PTAP Inclusion | 0.93 | 0.89–0.97 | Almost perfect correlation |
| Baseline vs Double-Weighting | 0.92 | 0.88–0.96 | Almost perfect correlation |

Jaccard Similarity for Cluster Membership

| Scenario Pair | Jaccard Similarity | Interpretation |
| --- | --- | --- |
| Baseline vs Grid (500m) | 0.85 | High stability |
| Baseline vs Grid (1km) | 0.82 | High stability |
| Baseline vs Grid (2km) | 0.80 | High stability |
| Baseline vs Rook Contiguity | 0.83 | High stability |
| Baseline vs k-NN (k=6) | 0.86 | High stability |
| Baseline vs Linear Scoring | 0.89 | Very high stability |
| Baseline vs Uniform Growth | 0.94 | Almost perfect stability |
| Baseline vs Ward-Specific Growth | 0.93 | Almost perfect stability |
| Baseline vs Proximity to Metro | 0.92 | Almost perfect stability |
| Baseline vs Zero-PTAP Inclusion | 0.91 | Almost perfect stability |
| Baseline vs Double-Weighting | 0.90 | Almost perfect stability |

Summary of Stability Measures

| Stability Measure | Overall Value | Interpretation |
| --- | --- | --- |
| Kendall's W (Primary Scenarios) | 0.86 | Strong concordance |
| Mean Spearman's ρ (All Scenarios) | 0.89 | Strong correlation |
| Mean Jaccard Similarity (All Scenarios) | 0.87 | High cluster stability |
| Minimum Stability (Any Measure) | 0.78 | Acceptable |
| Maximum Deviation in Classification | ±1 ward | Minimal |

Interpretation of Stability Thresholds:

- ≥0.90: Very High / Almost Perfect Stability

- 0.80–0.90: High Stability

- 0.70–0.80: Moderate Stability

- <0.70: Low Stability

Conclusion: The Stable High SMI Core classification (Al Batha, Manfuha) exhibits high stability across all analytical scenarios, with all stability measures exceeding 0.78. This confirms that the primary findings are robust to alternative methodological specifications. The maximum deviation across all scenarios was ±1 ward, confirming that the identified spatial patterns are not artifacts of analytical choices.